ASPEN GROVE CONDOMINIUM ASSOCIATION

WINTERIZATION INFORMATION

To Homeowners and Rental Companies

## FOR UNITS 3001-3132

- Shut off and drain the water lines when the unit is unoccupied throughout the year. While pipe freezes occur during the winter, we have experienced flooding in all seasons from icemaker and dishwasher plumbing connection failures. Your unit’s water shut off valve is located in the closet of the rear downstairs bedroom. It is either a copper handle or a square steel rod protruding through the floor. Turn the valve handle a quarter turn clockwise to shut off the water service to the unit. In the “off” position cold water in the system drains out the side of the valve. Open the taps in the kitchen and bathrooms to further drain the lines.

- Set the thermostat to a minimum of 55 degrees. Approximately half of the furnaces are equipped with a metal duct running from the furnace plenum to a hole in the utility room floor. This duct provides heat to the crawl space area and prevents the water and sewer mains from freezing.

Should your furnace not be equipped with a duct from the plenum to the floor, the unit has an electric heater in the crawl space that should be turned on from November through April. A switch on the circuit breaker panel located in the closet of the front downstairs bedroom should be switched on to activate the crawl space heater.

- Change the furnace filter at least once a year. The filter can be accessed by removing the lower sheet metal panel from the front of the appliance. Fall is a good time to perform this maintenance procedure along with changing batteries in smoke detectors.

- Open cabinet, bathroom, and laundry room doors to allow heat to circulate around uninsulated pipes under sinks and behind appliances near exterior walls.

# FOR TOWER UNITS 3133-3180

To prevent pipe freezes during the winter, please keep your thermostat at a minimum of 55 degrees and open cabinet and bathroom doors to allow heat to circulate around uninsulated pipes under sinks.

Should you have difficulty winterizing the unit give the CAMCO office a call and we’ll give you a hand.    (530) 587-3355     (800) 916-2262